# FLIGHT ANALYSIS REPORT

| Drone Name: | Tavas |
| --- | --- |
| MSN No: | 123 |
| Date: | 2026-04-07 |

__________________________________________________

# CRITICAL ALERTS

⚠️ Mechanical/Thrust Loss Failure

# Standard vs. Logged Parameters

| Parameter | Status | Standard | Logged |
| --- | --- | --- | --- |
| AHRS_TRIM_X | Changed | -0.0236823 | -0.0038239017594605684 |
| AHRS_TRIM_Y | Changed | 0.0212514 | 0.05083673447370529 |
| ATC_ACCEL_P_MAX | Changed | 90037.93 | 90037.9296875 |
| ATC_ACCEL_R_MAX | Changed | 134579.4 | 134579.40625 |
| ATC_ACCEL_Y_MAX | Changed | 15000.0 | 21000.0 |
| ATC_RAT_PIT_D | Changed | 0.0059933 | 0.005448500160127878 |
| ATC_RAT_PIT_I | Changed | 0.1207451 | 0.13416120409965515 |
| ATC_RAT_PIT_P | Changed | 0.1207451 | 0.13416120409965515 |
| ATC_RATE_Y_MAX | Changed | 90.0 | 0.0 |
| ATC_SLEW_YAW | Changed | 4000.0 | 6000.0 |
| BARO1_GND_PRESS | Changed | 90866.74 | 101409.4921875 |
| BATT_AMP_PERVLT | Changed | 22.008 | 66.02300262451172 |
| BATT_LOW_VOLT | Changed | 20.6 | 21.0 |
| COMPASS_DIA_X | Changed | 0.9718277 | 0.9994962811470032 |
| COMPASS_DIA_Y | Changed | 0.9973793 | 1.040884017944336 |
| COMPASS_DIA_Z | Changed | 1.082085 | 1.065704584121704 |
| COMPASS_ODI_X | Changed | -0.0034061 | 0.004146155435591936 |
| COMPASS_ODI_Y | Changed | -0.0053706 | -0.025395242497324944 |
| COMPASS_ODI_Z | Changed | -0.0285832 | -0.03937860205769539 |
| COMPASS_OFS_X | Changed | 63.01507 | 176.37379455566406 |
| COMPASS_OFS_Y | Changed | 53.37602 | 71.66236877441406 |
| COMPASS_OFS_Z | Changed | -155.6312 | -71.65877532958984 |
| FENCE_ALT_MAX | Changed | 120.0 | 100.0 |
| FENCE_RADIUS | Changed | 1500.0 | 300.0 |
| INS_ACC1_CALTEMP | Changed | 51.57005 | 53.019325256347656 |
| INS_ACC2_CALTEMP | Changed | 51.57005 | 53.50241470336914 |
| INS_ACC2OFFS_X | Changed | 0.0246186 | 0.09888699650764465 |
| INS_ACC2OFFS_Y | Changed | 0.0403881 | -0.00093028973788023 |
| INS_ACC2OFFS_Z | Changed | -0.2050692 | -0.22888322174549103 |
| INS_ACC2SCAL_X | Changed | 0.9988143 | 1.0006753206253052 |
| INS_ACC2SCAL_Y | Changed | 1.004205 | 1.0005404949188232 |
| INS_ACC2SCAL_Z | Changed | 1.007888 | 1.0036382675170898 |
| INS_ACCOFFS_X | Changed | -0.032412 | -0.04002758488059044 |
| INS_ACCOFFS_Y | Changed | -0.0028274 | 0.0007435931474901736 |
| INS_ACCOFFS_Z | Changed | -0.1662565 | -0.014494531787931919 |
| INS_ACCSCAL_X | Changed | 0.9998301 | 1.0012707710266113 |
| INS_ACCSCAL_Y | Changed | 1.005695 | 1.0019885301589966 |
| INS_ACCSCAL_Z | Changed | 1.00947 | 1.00838041305542 |
| INS_GYR1_CALTEMP | Changed | 34.66183 | 38.52656936645508 |
| INS_GYR2_CALTEMP | Changed | 34.66183 | 38.28502655029297 |
| INS_GYR2OFFS_X | Changed | 0.0071312 | 0.012557867914438248 |
| INS_GYR2OFFS_Y | Changed | -0.0151349 | -0.028099440038204193 |
| INS_GYR2OFFS_Z | Changed | -0.0104236 | -0.013845398090779781 |
| INS_GYROFFS_X | Changed | -0.014555 | -0.010152798146009445 |
| INS_GYROFFS_Y | Changed | -0.0307696 | -0.003984950482845306 |
| INS_GYROFFS_Z | Changed | 0.0057832 | 0.005545143969357014 |
| INS_HNTCH_ATT | Changed | 35.0 | 30.0 |
| INS_HNTCH_HMNCS | Changed | 7.0 | 3.0 |
| MAV1_EXT_STAT | Changed | 2.0 | 0.0 |
| MAV1_EXTRA1 | Changed | 4.0 | 0.0 |
| MAV1_EXTRA2 | Changed | 4.0 | 0.0 |
| MAV1_EXTRA3 | Changed | 2.0 | 0.0 |
| MAV1_POSITION | Changed | 2.0 | 0.0 |
| MAV1_RAW_SENS | Changed | 2.0 | 0.0 |
| MAV1_RC_CHAN | Changed | 2.0 | 0.0 |
| MIS_TOTAL | Changed | 75.0 | 1.0 |
| MOT_THST_HOVER | Changed | 0.3894711 | 0.3642388582229614 |
| RC11_TRIM | Changed | 1051.0 | 1951.0 |
| RC12_TRIM | Changed | 1051.0 | 1951.0 |
| RC7_TRIM | Changed | 1501.0 | 1951.0 |
| RC9_TRIM | Changed | 1861.0 | 1141.0 |
| STAT_BOOTCNT | Changed | 260.0 | 24.0 |
| STAT_FLTTIME | Changed | 66978.0 | 2821.0 |
| STAT_RUNTIME | Changed | 150595.0 | 8455.0 |

# Battery Voltage Range

Maximum Voltage: 25.84V

Minimum Voltage: 21.28V